Case Study

Modul 6: for loop

Kode Multiplier

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.numeric_std.all;

entity MultiplierDewa is

generic (

n : positive := 4

);

port (

dataA, dataB : IN std_logic_vector (n - 1 downto 0);

dataOut : OUT std_logic_vector (2*n - 1 downto 0)

);

end entity MultiplierDewa;

architecture rtl of MultiplierDewa is

begin

shift: process(dataA, dataB)

variable loopLen : integer := 0;

variable andTemp : std_logic := '0';

variable dataTemp : std_logic_vector (2*n - 1 downto 0) := (others => '0');

begin

-- dataTemp := dataIn;

loopLen := n - 1;

dataTemp := (others => '0');

for i in 0 to loopLen loop

for j in 0 to loopLen loop

dataTemp(i+j)  := dataTemp(i+j) xor (dataA(i) AND dataB(j));

dataTemp(i+j) := dataTemp(i+j) xor andTemp;

end loop;

end loop;

dataOut <= dataTemp;

end process shift;

end architecture rtl;

Kode Test Bench

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.numeric_std.all;

entity MultiplierDewa_tb is

end entity MultiplierDewa_tb;

architecture rtl of MultiplierDewa_tb is

constant n : positive := 8;

signal dataA, dataB : std_logic_vector (n - 1 downto 0);

signal dataOut: std_logic_vector (2*n - 1 downto 0) := (others => '0');

-- constant DELAY : time := 100 ps;

begin

UUT : entity work.MultiplierDewa

GENERIC MAP (n => n)

port map (dataA => dataA, dataB => dataB, dataOut => dataOut);

test1: process

begin

dataA <= "00101011";

dataB <= "11010010";

assert dataOut = "0001110110100110" report "jawaban salah" severity error;

wait for 100 ps;

dataA <= "11001100";

dataB <= "01010101";

assert dataOut = "0011110000111100" report "jawaban salah" severity error;

wait for 100 ps;

dataA <= "11110000";

dataB <= "00001111";

assert dataOut = "0000010101010000" report "jawaban salah" severity error;

wait for 100 ps;

dataA <= "00000001";

dataB <= "11111111";

assert dataOut = "0000000011111111" report "jawaban salah" severity error;

wait for 100 ps;

dataA <= "10101010";

dataB <= "01010101";

assert dataOut = "0010001000100010" report "jawaban salah" severity error;

wait for 100 ns;

wait;

end process test1;

end architecture rtl;

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.numeric_std.all;

entity MultiplierDewa_tb is

end entity MultiplierDewa_tb;

architecture rtl of MultiplierDewa_tb is

constant n : positive := 16;

signal dataA, dataB : std_logic_vector (n - 1 downto 0);

signal dataOut: std_logic_vector (2*n - 1 downto 0) := (others => '0');

-- constant DELAY : time := 100 ps;

begin

UUT : entity work.MultiplierDewa

GENERIC MAP (n => n)

port map (dataA => dataA, dataB => dataB, dataOut => dataOut);

test1: process

begin

dataA <= "1101101010110101";

dataB <= "0101010101010101";

assert dataOut = "00111000100011101100011101110001" report "jawaban salah" severity error;

wait for 100 ps;

dataA <= "1111111111111111";

dataB <= "0000000000000001";

assert dataOut = "00000000000000001111111111111111" report "jawaban salah" severity error;

wait for 100 ps;

dataA <= "0000111100001111";

dataB <= "1111000011110000";

assert dataOut = "00000101010100000000010101010000" report "jawaban salah" severity error;

wait for 100 ps;

dataA <= "0000000000000000";

dataB <= "1111111111111111";

assert dataOut = "00000000000000000000000000000000" report "jawaban salah" severity error;

wait for 100 ps;

dataA <= "1010101010101010";

dataB <= "0101010101010101";

assert dataOut = "00100010001000100010001000100010" report "jawaban salah" severity error;

wait for 100 ns;

wait;

end process test1;

end architecture rtl;